Basic 3D Antenna Library usage in ADS.

‘Johanson_3D AntennaLibrary_and_ADSexample’ contains the following files / directories.

- ‘Johanson_ADS_3DAnt_Lib_v1.zip’ is an archive of an Open Access encrypted EMPro Library containing 3D models of Johanson’s antenna catalog. It is unarchived in this directory as a library folder, ‘Johanson_ADS_3DAnt_Lib_v1’. You may unarchive this library in any convenient location in your file system. In the next steps, we will point ADS to this file location.

- ‘_3Dant_test_v2_wrk’ is an example ADS workspace.

  - Cell ‘0900AD47A2450-EB1SMA’ is the Eval Evaluation Board for this dual band 900M, 2.4G antenna. The rfpro cellview mounts the 3D antenna on the PCB and FEM simulates the assembly.

  - Cell ‘Ant_Test’ is a simple S-parameter test bench simulating the EM results from the rfpro simulation.

To access the 3D Antenna Library in ADS, from the ADS Main WindowFileManage LibrariesAdd Library

Navigate to where your ‘Johanson_ADS_3DAnt_Lib_v1’ directory is located, you will see Library added to Workspace:

From the ADS Main Window, open the ‘0900AD47A2450-EB1SMA’ rfpro cellview. Or, from LayoutToolsRFProOpen

To use 3D Components in RFPro. FileInsertEMPro 3D Component…

(0,0,0) of the Component will align with (0,0,0) of the Parent Design. Right Click the Component, either in the Design Tree or the Geometry WindowSpecify Orientation:

Basic Move and Rotation commands are available.

‘Advanced Mode’ is quite useful for specifying exact x,y,z placement location:

3D Components will only simulation using FEM Generation 2. In Simulator Options, choose FEMGeneration2.

Simulate Away!